# Playstation 4

https://pypi.org/project/pyPS4Controller

bluetoothctl is a core Linux Bluetooth functionality and it's the best way to connect your controller if you faced issues or errors with the GUI method.

# Open up your terminal and start up the tool by typing the command: sudo bluetoothctl # Enable the Bluetooth agent agent on discoverable on pairable on default-agent # Scan for devices scan on

This will start scanning for devices. The terminal should start listing scan results with all the bluetooth devices around you so pay some attention to the output because we need to find the controller.

While the command tool is scanning, press and hold both your Share and PS buttons on your controller. Keep holding the buttons at the same time until your controller starts flashing light.

Get the MAC address of the controller from the scan information.

connect CONTROLLER_MAC_ADDRESS

Done. If everything went well you should see a connection successful message appear on your terminal like so

Attempting to connect to AC:FD:93:14:25:D3

[CHG] Device AC:FD:93:14:25:D3 Connected: yes

[CHG] Device AC:FD:93:14:25:D3 ServicesResolved: yes

[CHG] Device AC:FD:93:14:25:D3 Paired: yes

Connection successful

We need to add this newly paired bluetooth device to the trusted list so next time the PS4 controller can automatically connect the Raspberry Pi. Type in the command:

trust CONTROLLER_MAC_ADDRESS

DONE! Your controller should be connected, paired and trusted.

The next time you press a button on the controller to wake it up, the controller should automatically connect to the Pi.

# Xbox

https://github.com/linusg/xbox360controller

pip3 install -U xbox360controller